Preface

Vide 13th amendment of the Electricity Supply Code, vide notification no. UPERC/Secy/Regulations/Supply Code/2018/206 dated 10 August 2018 amendment of supply code Hon’ble UPERC has ordered conversion of Single Point electricity connection provided to existing Multistoried buildings into multipoint connection.

Through Suo Moto proceedings  dated 17 May 2019, Hon’ble  UPERC has also brought into existence certain provisions which has been repealed by 13th amendment namely all new domestic multi-storied buildings and commercial multi-storied buildings barring multiplex, marriage halls and centralized air-conditioned commercial multi-storied buildings shall be given individual connections on multi-point in future, etc as annexed (Suo moto proceedings in implementation of 13th Amendment of Electricity Supply Code, 2005).

Since, majority of the domestic consumers in multi-storied buildings are having single electricity distribution infrastructure meant for supply of Utility power as well as DG power, and since supply code amendment provides for conversion of existing single point connection into multi-point connection by adopting technological solution.

In compliance to the directives issued by Hon’ble UPERC, PVVNL intends to work upon various methodologies to achieve the desired outcome as stipulated in the directives. PVVNL also intends to release guidelines to various developers to adopt such solutions in their ongoing and upcoming projects in order to implement multi-point connections.

It has been deliberated upon by the committee that multi-point connection can be effected in such multistorey complexes with minimal infrastructure changes by adopting proven technology solutions. Such solutions were observed working satisfactorily during survey of various buildings under PVVNL. It is possible through technological intervention and modification in the IT hardware like DCU and gateway and software components like HES and MDM.

Existing smart metering components being used in such solutions such as DCU, Gateway and meters with additional feature like handling dual supply (from utility and DG) triggered by a supply based signal to change register of meter. Further, the solution has dual HES and separate MDM for Utility and DG supply.

In order to achieve error free and accurate system detailed functionalities has been carved out in the Section Functional Requirements in the Implementation guidelines for deployment of multipoint dual source metering system.

PVVNL is looking forward to empanel Dual source metering solution providers who fulfils and adheres to the detailed implementation guidelines prescribed by PVVNL for dual source metering infrastructure implementation, maintenance and management.

Responsibility of Dual Source Metering Solution provider

Such AMR solution providers will deploy, manage and maintain AMR network at building level.

- Further, Dual source metering solution providers need to provide centralized monitoring, NOC, generate alerts and reports as desired by PVVNL. It is intended to empanel only those solution providers who have technical capability and experience to provide Dual HES for dual metering including deployment of Server as well as can maintain uptime of server as per utility defined SLA. The empaneled solution provider to be capable of integrating / providing data of PVVNL’s supply to PVVNL’s billing system in desired format and should be capable to integrate with PVVNL’s revenue system.

- Empaneled service provider needs to sign tripartite agreement with PVVNL and Developer/ RWA.

- The empaneled solution provider needs to supply, install and commission all the equipment, i.e. Servers, switches / routers and other necessary items.

- The Dual metering infrastructure solution provider shall undertake the operation and maintenance of the complete AMI system for total contract period subsequent to its commissioning. The operation & maintenance charges will include replacement of defective equipment / material supplied by the solution provider, carrying out regular and periodic field activities required for ensuring proper health of the deployed infrastructure, etc.

- The solution provider shall also provide adequate manpower to maintain the system in the field for duration of contract period from the date of successful installation and commissioning of the Multi Point system.